# AAE 590 ACA: FINAL PROJECT PROPOSAL

Mustakeen Bari

# Motivation and Context

In designing controls for orbital dynamics, a popular method for solving Two-Point Boundary Value Problems (TPBVP), as taught in class, is the shooting method. The method works by converting the boundary value problem into an initial value problem. By guessing initial conditions and propagating a system using ordinary differential equations, the method “shoots” a trajectory with the goal of meeting a desired final boundary condition. The propagated final state of the system is compared to the desired boundary conditions, and the difference is used to inform the initial conditions of subsequent iterations. Over enough repetitions, the method calculates a trajectory, the end of which approximates the desired boundary condition to within a predetermined tolerance.

While capable of handling simpler trajectories, the shooting method often fails in non-trivial scenarios and is extremely sensitive to the quality of the initial guesses. Poor initial guesses make the method more inefficient and, in many cases, outright prevent the calculation from converging. The shooting method also struggles to converge when the trajectory has sudden (or “sharp”) shifts or path constraints. Additionally, the nature of the algorithm is computationally expensive due to requiring an indefinite number of iterations as well as having to optimize all the points over the trajectory.

Many modifications and alternative methods of solving TPBVP exist to address these issues, one of which is the use of Bezier Curves as represented by Bernstein basis polynomials. The curves have specific properties that make them particularly well-suited for trajectory optimization. They are smooth and multiple simple Bezier segments can be linked together (collocated into a type of “B-Spline”) to form complex trajectories. Most importantly, however, in the Bernstein polynomial form, each curve segment is defined by only a handful of control points (four in the case of cubic) and a continuous parameter “t”. These points include the start and end of each line segment through which the trajectory passes, i.e. interpolates. However, the control points between each Bezier segment are not interpolated through and instead act as weights to influence the shape of the curve.

In the context of TPBVP, converting the trajectory representation to the basis polynomial form can utilize these properties in the optimization. Since the Bezier Curves are defined by a few parameters, this reduces the complex optimization of all points over a trajectory into a problem of optimizing only a handful of control points. The inherent start and end points of the curves enable a more robust optimization process for identifying initial conditions, potentially addressing the initial condition sensitivity of the shooting method.

While other collocation methods exist such as the Hermite-Simpson method, for this project I intend to focus on the collocation of Bezier Curves due to its applicability to path constrained problems. For the scope of the class, I plan to utilize Bezier Curves to calculate trajectories for an Earth-to-Mars orbit transfer and compare the results against the traditional shooting method. If time permits, I would like to extend the application to a Rendezvous & Proximity Operation or collision avoidance scenario.

# Literature Survey

The following are the three most relevant papers I have found adjacent to the problem space I intend to work in:

- Choi, Daegyun, et al., “A STUDY ON EFFECTIVE INITIAL GUESS FINDING METHOD BASED ON BEZIER CURVES: ORBIT DETERMINATION APPLICATIONS”, arXiv, 2025

  - This paper serves as the foundation for my project. It discusses an approach of transforming an orbit determination problem into a Bezier form to make better initial guesses, applying it to different kinds of orbits, and finally comparing it against the shooting method for each. It finds that the proposed method can calculate trajectories in certain scenarios even when the shooting method fails.

  - While the paper discusses orbit determination, it does not address a specific orbital transfer problem and is not a controls application.

- Howell, Lauren, et al., “Spline Trajectory Algorithm Development: Bézier Curve Control Point Generation for UAVs”, AIAA, 2016

  - This paper uses Bezier Curves for the trajectory optimization of UAVs in a Euclidean space. It demonstrates an algorithm that generates B-Splines to inform autonomous flight. Unlike the previous paper, this is a controls application.

  - While the paper discusses controls for UAVs in the context of drone flight, it does not address a scenario involving spaceflight or orbit propagation.

- Shin, Vladimir, et al., “Bézier Curves for Smooth Entry into Elliptic Orbits”, IJASS, 2024

  - As the title implies, this paper discusses a path planning approach using Bezier Curves to smoothly transition an off-orbit aerospace vehicle into an elliptical orbit. This is perhaps the closest to my intended project, being both a controls and orbital dynamics application.

  - While the paper does discuss transitioning into orbits, it does not address a restricted three-body problem like an Earth-to-Mars orbit transfer.

I have provided some additional papers that provide more context or computational methods relevant to the trajectory calculations:

- Vinayak, Akhil, et al., “A novel Bezier curve control point search algorithm for autonomous navigation using N-order polynomial search with boundary conditions”, IEEE ITSC, 2021

  - Discusses control point determination for Bezier Curves used in autonomous driving.

- Choi, Ji-wung, et al., “Bezier Curve for Trajectory Guidance”, UC Santa Cruz, 2008

  - Discusses two pathing algorithms using Bezier Curves with path constraints for autonomous driving.

- Schneider, Philip, “AN ALGORITHM FOR AUTOMATICALLY FITTING DIGITIZED CURVES”, IMPA, 1990

  - Discusses a popular algorithm that fits Bezier Curves to an existing set of points in a 2D space, which can be extended for 3D applications.

- Hanik, Martin, et al., “De Casteljau’s Algorithm in Geometric Data Analysis: Theory and Application”, arXiv, 2024

  - Discusses, in depth, de Casteljau’s algorithm for formulating Bezier Curves and splitting along the curve itself.