# 1. Introduction

Our project compares two numerical approaches for solving optimal trajectory design problems in cislunar space: indirect shooting (based on Pontryagin’s Maximum Principle) and direct collocation (using Bézier curves with IPOPT). Both methods are rooted in the optimal control theory taught in AAE 568.

This document traces each core concept from the lecture notes to its concrete implementation in our project code, demonstrating how the course material directly underpins the trajectory optimization work.

# 2. Problem Formulation as an Optimal Control Problem

## 2.1 The Bolza Problem (Lecture Notes, pp. 141–142)

AAE 568 introduces the general optimal control problem in Bolza form:

min  J = φ(x(tf), tf) + ∫ L(x, u, t) dt

subject to  ẋ = f(x, u, t),  x(t₀) = x₀

Our project instantiates this framework with a minimum-energy trajectory transfer:

- Dynamics f(x, u): the Circular Restricted Three-Body Problem (CR3BP), a nonlinear 6D system modeling the Earth–Moon gravitational field in a rotating frame. The equations include Coriolis and centrifugal acceleration terms.

- State x = [x, y, z, vx, vy, vz]: spacecraft position and velocity in the rotating frame.

- Control u = [ux, uy, uz]: low-thrust acceleration applied by the propulsion system.

- Running cost L(x, u) = |u|² (minimum energy), or L = |u| (minimum fuel, with smoothed bang-bang).

- Terminal cost φ = 0 (Lagrange problem) with fixed boundary conditions at LEO departure and NRHO arrival.

This is exactly the Bolza formulation from the lectures, instantiated on a physical astrodynamics problem.

## 2.2 Cost Functions (Lecture Notes, p. 142)

The lecture notes present three classic performance measures. Our project implements all three:

| Cost Type | Lecture Definition | Project Implementation |
| --- | --- | --- |
| Min. Energy | J = ∫ (xᵀQx + uᵀRu) dt | J = ∫ ¦u¦² dt (with Q = 0, R = I) |
| Min. Time | J = ∫ 1 dt = tf – t₀ | Free final time tf as decision variable; Hamiltonian transversality H(tf) = 0 |
| Min. Fuel | J = ∫ ¦u¦ dt | Smoothed bang-bang via tanh(S/ρ) with homotopy continuation on ρ |

# 3. Calculus of Variations and Euler-Lagrange Equations

## 3.1 Augmented Cost and Hamiltonian (Lecture Notes, pp. 143–145)

The core technique from the lectures is to adjoin the dynamics constraint using a Lagrange multiplier (costate) λ(t):

Ja = φ(x(tf)) + ∫ [ℓ(x, u, t) + λᵀ(f(x, u, t) – ẋ)] dt

Defining the Hamiltonian as:

ℋ(x, u, λ, t) = ℓ(x, u, t) + λᵀ f(x, u, t)

In our project, for the minimum-energy case with CR3BP dynamics, the Hamiltonian takes the explicit form:

ℋ = |u|² + λrᵀ v + λvᵀ (fCR3BP(x) + u)

This is implemented in dynamics.py as the cr3bp_controlled_ode function (lines 164–229), which propagates the 12D combined state–costate system.

## 3.2 Euler-Lagrange Necessary Conditions (Lecture Notes, pp. 146–147)

Setting the first variation δJ to zero yields three sets of necessary conditions:

- Costate equation: λ̇ = –(∂ℋ/∂x)ᵀ  —  The costate must satisfy this backward differential equation.

- Transversality: λ(tf) = (∂φ/∂x)ᵀ|tf  —  Terminal costate determined by terminal cost.

- Stationarity: ∂ℋ/∂u = 0  —  The control must minimize the Hamiltonian.

These three conditions appear explicitly in our indirect shooting code:

### Costate Equations in dynamics.py

For the two-body case (lines 31–59), the costate derivatives are derived analytically from the Hamiltonian. The gravity gradient tensor ∂f/∂x appears as:

λ̇r = –[μ(3rrᵀ/|r|⁵ – I/|r|³)] λv,    λ̇v = –λr

For the 3D CR3BP (lines 164–229), the gravity gradient becomes the full Hessian of the pseudo-potential, including cross-coupling terms (U_xx, U_xy, U_xz, U_yy, U_yz, U_zz) and Coriolis coupling between velocity and position costates.

### Stationarity Condition

For minimum energy, applying ∂ℋ/∂u = 0:

2u + λv = 0   ⇒   u* = –½ λv

This optimal control law appears in the code as u = -0.5 * lam_v (dynamics.py, lines 46–47 for two-body, line 179 for CR3BP). This is a direct application of the Euler-Lagrange stationarity condition from the lectures.

# 4. Pontryagin’s Minimum Principle

## 4.1 PMP with Control Constraints (Lecture Notes, pp. 153–160)

When the control is constrained to an admissible set U, the Euler-Lagrange condition ∂ℋ/∂u = 0 is replaced by the Minimum Principle:

ℋ(x*, u*, λ*, t) ≤ ℋ(x*, u, λ*, t)    ∀ u ∈ U

Our project applies this in multiple settings:

### Bang-Bang Control (Minimum Time)

For the minimum-time problem with bounded thrust |u| ≤ u_max, the lecture notes show that PMP yields a bang-bang control structure:

u*(t) = –umax · λv / |λv|

This is implemented in dynamics.py lines 62–89 (two_body_min_time_ode). The thrust direction is always anti-aligned with the velocity costate, and the magnitude is at the bound — exactly the bang-bang structure derived in the lectures.

### Smoothed Bang-Bang (Minimum Fuel)

For minimum fuel, the switching function S = |λ_v| – 1 determines thrust on/off. Because the discontinuous switch makes numerical integration difficult, we apply the smoothing technique:

T = (umax/2)(1 + tanh(S/ρ))

The smoothing parameter ρ is driven toward zero through a homotopy continuation schedule (shooting.py, lines 170–193), gradually recovering the true bang-bang solution. This connects to the lecture discussion of inequality constraints on the control (pp. 161–166).

# 5. Two-Point Boundary Value Problem and Shooting Method

## 5.1 The TPBVP Structure (Lecture Notes, pp. 147–148)

The Euler-Lagrange conditions transform the optimal control problem into a TPBVP: the state equations propagate forward from known initial conditions x(t₀) = x₀, while the costate equations have terminal conditions λ(t_f) specified by transversality. This split-boundary structure — “forward integration for states, backward integration for costates” — is a central theme of the course.

## 5.2 Indirect Shooting (Lecture Notes, pp. 186–192)

The lectures present MATLAB’s bvp4c as a standard numerical tool for solving the TPBVP. Our project generalizes this approach using a shooting method:

- Guess initial costates λ(t₀).

- Propagate the combined 8D (two-body) or 12D (CR3BP) system forward from t₀ to t_f.

- Evaluate residual: terminal state misses the target x(t_f) – x_target.

- Iterate using Newton’s method (fsolve) to drive the residual to zero.

This is implemented in shooting.py, with three variants corresponding to the three cost types from the lectures: solve_min_energy (lines 133–148), solve_min_time (lines 151–166), and solve_min_fuel (lines 169–193).

## 5.3 Free Final Time and Hamiltonian Transversality (Lecture Notes, pp. 150–152)

For the open-end-time case, the lectures derive the additional transversality condition:

(d/dt) Ja|tf = (∂φ/∂t + ℋ)|tf = 0

When φ is independent of t_f and the system is autonomous, this reduces to H(t_f) = 0. In our minimum-time shooting (shooting.py, lines 58–107), we include t_f as an unknown alongside the initial costates, and the Hamiltonian residual H_f is part of the shooting function — exactly matching the open-end-time transversality condition from the notes.

# 6. Direct Collocation — From Variational Calculus to NLP

## 6.1 Constrained Optimization and Lagrange Multipliers (Lecture Notes, pp. 133–140)

Before introducing dynamic optimization, the course covers static constrained optimization: minimize J(u) subject to f(x, u) = 0, using Lagrange multipliers to form the augmented cost J_a = J + λᵀ f. The direct collocation approach in our project applies the same principle, but at discrete collocation points.

Instead of deriving continuous necessary conditions first and then discretizing (the indirect approach), direct collocation discretizes the problem first and solves the resulting large-scale NLP directly. The dynamics constraints f(x, u) = 0 become equality constraints in the NLP, and the NLP solver (IPOPT) internally manages the KKT multipliers — the discrete analogues of the costates λ(t).

## 6.2 Bézier Curve Parameterization

We parameterize the state trajectory using Bézier curves (Bernstein polynomials), with control points as decision variables. The key advantages that connect to course material:

- Dynamics defects as equality constraints: At Gauss-Legendre collocation points, we enforce ẋ_Bézier(t_k) = f(x(t_k), u(t_k)), where the left side comes from the analytic derivative of the Bézier curve. These are exactly the equality constraints of the constrained optimization formulation from the lectures.

- Objective as quadrature: The running cost ∫|u|² dt is approximated by Gauss-Legendre quadrature: J ≈ Σ w_k |u_k|² Δt. This discretizes the Lagrange cost from the Bolza formulation.

- Boundary conditions: The first control point of the first segment equals x₀, and the last control point of the last segment equals x_f. These enforce the boundary conditions of the optimal control problem.

## 6.3 NLP Solution via IPOPT

The resulting NLP (ipopt_collocation.py and leo_to_nrho_cr3bp.py) is solved by IPOPT, an interior-point method. IPOPT internally solves the KKT system, which is the discrete version of the Euler-Lagrange equations: the KKT multipliers on the dynamics defect constraints are the numerical counterparts of the costates λ(t) that appear in the continuous theory.

CasADi provides exact sparse Jacobians and Hessians via automatic differentiation, which is essential because the gravity gradient tensor of the CR3BP (the same ∂f/∂x matrix that appears in the costate equations) also appears in the KKT system.

# 7. Mesh Refinement and Warm-Starting

A recurring theme in the numerical methods discussion (Lecture Notes, pp. 186–196) is the sensitivity of shooting methods to initial guesses. Our project addresses this through a coarse-to-fine mesh refinement cascade:

- Start with a coarse discretization (e.g., 20 segments) and loose tolerance.

- Once converged, interpolate the solution to a finer mesh (40, then 60 segments).

- Warm-start IPOPT with the interpolated solution from the previous level.

This directly addresses the course’s emphasis on the importance of good initial guesses for TPBVP solvers (bvp4c summary, p. 196). Additionally, we warm-start the indirect shooting from the direct collocation solution, using the relationship u* = –½λ_v to estimate initial costates from the IPOPT control solution.

# 8. Summary: Mapping Course Concepts to Project

The following table provides a comprehensive mapping between AAE 568 lecture topics and their implementation in the project:

| AAE 568 Concept | Lecture Reference | Project Implementation |
| --- | --- | --- |
| Bolza problem formulation | pp. 141–142 | Min-energy LEO→NRHO transfer in CR3BP |
| Hamiltonian & augmented cost | pp. 143–145 | cr3bp_controlled_ode in dynamics.py |
| Euler-Lagrange / costate equations | pp. 146–147 | Costate ODEs with gravity gradient tensor |
| Stationarity condition (∂H/∂u = 0) | p. 147 | Optimal control law u* = –½λ_v |
| Open-end-time transversality (H = 0) | pp. 150–152 | H_f residual in min-time shooting |
| Pontryagin’s Minimum Principle | pp. 153–160 | Bang-bang thrust direction for min-time |
| Inequality constraints (bounded control) | pp. 161–166 | Smoothed bang-bang with homotopy for min-fuel |
| TPBVP structure | pp. 147–148 | Forward state / terminal costate split |
| Numerical TPBVP solution (bvp4c) | pp. 186–196 | Indirect shooting with fsolve |
| Constrained optimization / Lagrange mult. | pp. 133–140 | NLP with dynamics defect constraints (IPOPT) |
| Quadratic cost (LQR-like) | p. 142 | ∫¦u¦²dt objective with Gauss-Legendre quadrature |
| Dynamic programming principle | pp. 197–206 | Mesh refinement cascade (coarse→fine) |
| Sufficient conditions (Legendre-Clebsch) | p. 152 | H_uu = 2I > 0 (convex in u for min-energy) |

# 9. Conclusion

This project is fundamentally a controls project: it applies nearly every major topic from AAE 568 to a real astrodynamics problem. The Bolza formulation, Hamiltonian, Euler-Lagrange equations, Pontryagin’s Minimum Principle, TPBVP shooting, transversality conditions, constrained optimization, and numerical methods all appear directly in our codebase. The comparison between indirect shooting and direct collocation provides a side-by-side view of the two major families of numerical optimal control methods — one deriving optimality conditions first and then solving the resulting TPBVP, the other discretizing the problem into a constrained NLP — both ultimately resting on the same variational principles taught in the course.